Simulation ALBATROSS II

| Date | 23.10.2026 |
| --- | --- |
| Participants | Diego Grünberger, Ciel Frey |

## Configuration

Rocket

Total mass dry [kg]: 11,894

Total mass wet [kg]: 16,968

CG from tip wet [m]: 1,324

CP from tip at rail exit [m]: 1,557 (OpenRocket); 1,652 (RocketPy)

Stability at rail exit (@ 2 m/s wind) [cal]: 1,84 (OpenRocket); 2,5 (RocketPy)

Thrust to weight ratio at rail exit: 16,48

Motor: 9977M2245-P

Fin span [m]: 0,328

Fineness ratio: 17,26

Launch rail

Length [m]: 7

Heading [°]: 40

Inclination [°]: 3

## Simulation Settings

Simulation 1

Standard OpenRocket environment settings (2 m/s surface wind speed, 0,2 m/s standard deviation, wind direction 90°)

Landing positions will be simulated for 0-10 m/s wind speed.

Simulation 2

real weather data from launch site

Surface wind speed: 3,31 m/s

## Results

Environment independent results:

|  | OpenRocket | RocketPy |
| --- | --- | --- |
| Rail departure time [s] | 0,296 | 0,287 |
| Velocity at rail exit [m/s] | 42,1 | 40,7 |
| Maximum speed [m/s] | 547 | 544,2 |
| Maximum Mach Number | 1,63 | 1,63 |
| Maximum acceleration [m/s²] | 183 | 182,2 |
| Maximum Gs [g] | 18,7 | 18,58 |

Simulation 1 (2 m/s wind speed):

|  | OpenRocket | RocketPy |
| --- | --- | --- |
| Apogee [m] | 6307 | 6356 |
| Impact speed [m/s] | 5,72 | 5,55 |
| Apogee time [s] | 32 | 32 |
| Total flight time [s] | 300 | 300 |

Simulation 2 (real weather data):

|  | OpenRocket | RocketPy |
| --- | --- | --- |
| Apogee [m] | 6298 | 6277 |
| Impact speed [m/s] | 6,84 | 5,48 |
| Apogee time [s] | 32 | 32 |
| Total flight time [s] | 301 | 300 |

## Stability plot

OpenRocket

RocketPy

## Total motion over time

## Trajectory

Simulation 1 (2 m/s wind speed)

Simulation 2 (real weather data)

## Landing positions

Simulation 1 (0-10 m/s wind speed)

Nominal furthest west for 10 m/s; furthest east for 0 m/s

No main furthest west for 10 m/s; furthest east for 0 m/s

Ballistic furthest west for 0 m/s; furthest east for 10 m/s

Classified as “suboptimal” since it lands in a forest if it flies ballistic.

OpenRocket

Landing distance from launch rail

nominal [m]: 487 - 1276

no main [m]: 485 - 926

ballistic [m]: 937 - 2126

RocketPy

Landing distance from launch rail

nominal [m]: 489 - 902

no main [m]: 485 - 677

ballistic [m]: 951 - 2938

Simulation 2 (real weather data)

Heading [°]: 40, 50, 60

Inclination [°]: 3, 4, 5

Classified as “suboptimal” since it lands in a forest if it flies nominal/with no main.

OpenRocket

Landing distance from launch rail

nominal [m]: 3194 - 3554

no main [m]: 2969 - 3338

ballistic [m]: 152 - 658

RocketPy

Some flights classified as “suboptimal” since it lands in a forest if it flies nominal/with no main.

Landing distance from launch rail

nominal [m]: 2782 - 3149

no main [m]: 2540 - 2910

ballistic [m]: 242 - 835